PC →μC

| Command |  | Function | # | Argument(s) |
| --- | --- | --- | --- | --- |
| O | A | Set Accelerometer Offsets | 3 | %04x<int16_t offsAccX>,%04x<int16_t offsAccY>,%04x<int16_t offsAccZ> |
|  | G | Set Gyroscope Offsets | 3 | %04x<int16_t offsGyrX>,%04x<int16_t offsGyrY>,%04x<int16_t offsGyrZ> |
| o | A | Request Accelerometer Offsets | - |  |
|  | G | Request Gyroscope Offsets | - |  |
| R | A | Set Acc. Offset Corr. On/Off | 1 | %d<int accOffsCorr > |
|  | G | Set Gyro Offset Corr. On/Off | 1 | %d<int gyrOffsCorr > |
| r | A | Request Acc. Offset Corr. On/Off | - |  |
|  | G | Request Gyro Offset Corr. On/Off | - |  |
| G | A | Set Accelerometer Gain Correction | 3 | %04x<uint16_t gainAccX>,%04x<uint16_t gainAccY>,%04x<uint16_t gainAccZ> |
|  | G | Set Gyroscope Gain Correction | 3 | %04x<uint16_t gainAccX>,%04x<uint16_t gainAccY>,%04x<uint16_t gainAccZ> |
| g | A | Request Acc. Gain Correction | - |  |
|  | G | Request Gyro Gain Correction | - |  |
| Q | A | Set Acc. Gain Correction On/Off | 1 | %d<int accGainCorr > |
|  | G | Set Gyro Gain Correction On/Off | 1 | %d<int gyrGainCorr > |
| q | A | Request Acc. Gain Correction On/Off | - |  |
|  | G | Request Gyro Gain Correction On/Off | - |  |
| C |  | Set IMU Configuration | 4 | %02x<uint8_t accODRLPF>,%02x< uint8_t accFS>, %02x< uint8_t gyrODRLPF>, %02x< uint8_t gyrFSHPF> |
| c |  | Request Configuration | - |  |
| P |  | Set Measurement Period | 1 | %04x<uint16_t Tsmp> |
| p |  | Request Measurement Period | - |  |
| F | W | Write Configuration to Data Flash | - |  |
|  | X | Erasing Data Flash | - |  |
| f |  | Request Data Flash State | - |  |
| A | X | Reset Integration Process | - |  |
| T | S | Set IMU Temperature SetPoint | 1 | %04x<int16_t tempSetp> |
| t | S | Request IMU Temperature SetPoint | - |  |

μC → PC

| Command |  | Function | # | Argument(s) |
| --- | --- | --- | --- | --- |
| D |  | Inertial Measurement Data | 6 | %04x<int16_t AccX>,%04x<int16_t AccY>,%04x<int16_t AccZ>, %04x< int16_t GyrX>,%04x< int16_t GyrY>,%04x< int16_t GyrZ> |
| T |  | Temperature Measurement | 1 | %04x<int16_t Temp> |
| A |  | Angle Data | 3 | %04x<int16_t angleX>,%04x<int16_t angleY>,%04x<int16_t angleZ> |
| V |  | Velocity Data | 3 | %04x<int16_t velX>,%04x<int16_t velY>,%04x<int16_t velZ> |
| S |  | Position Data | 3 | %04x<int16_t posX>,%04x<int16_t posY>,%04x<int16_t posZ> |
| L |  | Linear Acceleration and Gravity Vector | 6 | %04x<int16_t LaccX>,%04x<int16_t LaccY>,%04x<int16_t LaccZ>, %04x< int16_t GrVecX>,%04x< int16_t GrVecY>,%04x< int16_t GrVecZ> |
| O | A | Accelerometer Offsets | 3 | %04x<int16_t offsAccX>,%04x<int16_t offsAccY>,%04x<int16_t offsAccZ> |
|  | G | Gyroscope Offsets | 3 | %04x<int16_t offsGyrX>,%04x<int16_t offsGyrY>,%04x<int16_t offsGyrZ> |
| R | A | Send Acc. Offset Corr. On/Off | 1 | %d<int accOffsCorr > |
|  | G | Send Gyro Offset Corr. On/Off | 1 | %d<int gyrOffsCorr > |
| G | A | Accelerometer Gain Corrections | 3 | %04x<int16_gainAccX>,%04x<int16_t gainAccY>,%04x<int16_t gainAccZ> |
|  | G | Gyroscope Gain Corrections | 3 | %04x<int16_t gainGyrX>,%04x<int16_t gainGyrY>,%04x<int16_t gainGyrZ> |
| Q | A | Send Acc. Gain Corr. On/Off | 1 | %d<int accGainCorr > |
|  | G | Send Gyro Gain Corr. On/Off | 1 | %d<int gyrGainCorr > |
| C |  | Send Configuration | 4 | %02x<uint8_t accODRLPF>,%02x< uint8_t accFS>, %02x< uint8_t gyrODRLPF>,%02x< uint8_t gyrFSHPF> |
| P |  | Send Measurement Period | 1 | %04x<uint16_t Tsmp> |
| E |  | Error Reporting | 2 | %04x<uint16_t errSource>,%04x<uint16_t errInfo> |
| M |  | Message | 1 | %s<char message[max 128]> |
| F | C | Write to Data Flash Completed | - |  |
|  | E | Write to Data Flash Error | - |  |
| T | S | Send IMU Temperature SetPoint | 1 | %04x<int16_t tempSetp> |