Setting up AMS330LHH IMU module to periodic measurement

Setting up accelerometer (ACC)

Parameters

- Output Data Rate (ODR)	CTRL1_XL: ODR_XL[3:0]

- Low Pass Filter corner frequency	CTRL1_XL: LPF2_XL_EN; CTRL8_XL: HPCF_XL_[2:0]

- Full Scale	CTRL1_XL: FS[1:0]_XL

Setting up gyroscope (GYR)

Parameters

- Output Data Rate (ODR)	CTRL2_G: ODR_G[3:0]

- Low Pass Filter corner frequency	CTRL4_C: LPF1_SEL_G | CTRL6_C: FTYPE[2:0]

- High Pass Filter corner frequency	CTRL7_G: HPM_G[1:0]

- High Pass Filter enable	CTRL7_G: HP_EN_G

- Full Scale	CTRL2_G: FS[1:0]_G | FS_125 | FS_4000

Initialization data

typedef struct _ASM330LHH_INIT

{

uint8_t accODRLPF;		// Output Data Rate and  LP Filter settings for Accelerometer

uint8_t accFS;			// Full Scale selection for Accelerometer

uint8_t gyrODRLPF;		// Output Data Rate and  LP Filter settings for Gyroscope

uint8_t gyrFSHPF;		// Full Scale and HP Filter selection for Gyroscope

} ASM330LHH_INIT;

IMU Operation States

ASM330LHH_STATE_INIT	0

ASM330LHH_STATE _RESET	1

ASM330LHH_STATE _MEAS	2