Communication frame structure

| Sync characters |  | Message identifier |  | Length (N) |  | Payload (N bytes – N: integer multiple of 4) |  |  | CRC-32 |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| 0 | 1 | 0 | 1 | 0 | 1 | 0 |  | N-1 |  |  |  |  |
| 0 | 1 | 2 | 3 | 4 | 5 | 6 |  |  | 6+N | 7+N | 8+N | 9+N |
| 0xCA | 0xE5 | Range over which CRC is calculated (size: integer multiple of 4) |  |  |  |  |  |  |  |  |  |  |

Sync field: 0xE5CA = 0b1100010111001010

Idle message

| 0xCA | 0xE5 | 0x00 | 0x00 | 0x0000 | CRC-32 |
| --- | --- | --- | --- | --- | --- |

uint8_t msgBuf[NN] – az aktuálisan kapott üzenetet tartalmazza

msgCnt – az üzenethez tartozó aktuálisan kapott byte-ok száma, alapértéke: -2

msgCnt == -2 – az első szikronizációs karakter keresése

msgCnt == -1 – a második szinkronizációs karakter azonosítása

msgCnt == 0 – az első üzenet-byte fogadása

az msgCnt 2. és 3. byte ad meg egy üzenet hosszt, legyen ez N

beolvasunk N+4 byte-ot, az utolsó 4 a kapott CRC érték.

A 4. byte-tól az N+4-ig kiszámítjuk a CRC-32 értéket, ezt hasonlítjuk össze a kapott CRC-vel.

PC →μC

|  | typedef struct _PCCMD_SETINT3D { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (12) int16_t X; // X Data int16_t Y; // Y Data int16_t Z; // Z Data int16_t res; // reserved uint32_t CRC; // CRC-32 } PCCMD_SETINT3D; | Applicable for - OA – Set Accelerometer Offsets - OG – Set Gyroscope Offsets |
| --- | --- | --- |
|  | typedef struct _PCCMD_SETUINT3D { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (12) uint16_t X; // X Data uint16_t Y; // Y Data uint16_t Z; // Z Data int16_t res; // reserved uint32_t CRC; // CRC-32 } PCCMD_SETUINT3D; | Applicable for - GA – Set Accelerometer Gain Correction - GG – Set Gyroscope Gain Correction |
|  | typedef struct _PCCMD_SETUINT { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (8) uint16_t Data; // Data int16_t res; // reserved uint32_t CRC; // CRC-32 } PCCMD_SETUINT; | Applicable for - RA – Set Acc. Offset Corr. On/Off - RG – Set Gyro Offset Corr. On/Off - QA – Set Acc. Gain Correction On/Off - QG – Set Gyro Gain Correction On/Off - P – Set Measurement Period |

|  | typedef struct _PCCMD_REQPARS { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (4) uint32_t CRC; // CRC-32 } PCCMD_REQREQPARS; | Applicable for - oA – Request Accelerometer Offsets - oG – Request Gyroscope Offsets - rA – Request Acc. Offset Corr. On/Off - rG – Request Gyro Offset Corr. On/Off - qA – Request Acc. Gain Correction On/Off - qG – Request Gyro Gain Correction On/Off - FC – Write Configuration to Data Flash - FE – Erasing Data Flash - c – Write to Data Flash Completed - p – Write to Data Flash Error |
| --- | --- | --- |
|  | typedef struct _PCCMD_IMUCONF { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (8) uint8_t B0; // Byte 0 uint8_t B1; // Byte 1 uint8_t B2; // Byte 2 uint8_t B3; // Byte 3 uint32_t CRC; // CRC-32 } PCCMD_IMUCONF; | Applicable for - C – Set IMU Configuration |

Payload length: bytes number of payload without CRC – multiples of 4.

μC → PC

|  | typedef struct _IMU_DATA { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (48) int16_t accX; // Accelerometer Measurement X int16_t accY; // Accelerometer Measurement Y int16_t accZ; // Accelerometer Measurement Z int16_t gyrX; // Gyroscope Measurement X int16_t gyrY; // Gyroscope Measurement Y int16_t gyrZ; // Gyroscope Measurement Z int16_t angleX; // Angle X Data int16_t angleY; // Angle Y Data int16_t angleZ; // Angle Z Data int16_t velX; // Velocity X Data int16_t velY; // Velocity Y Data int16_t velZ; // Velocity Z Data int16_t posX; // Position X Data int16_t posY; // Position Y Data int16_t posZ; // Position Z Data int16_t laccX; // Linear Acceleration X Data int16_t laccY; // Linear Acceleration Y Data int16_t laccZ; // Linear Acceleration Z Data int16_t grvecX; // Gravity Vector X Data int16_t grvecY; // Gravity Vector Y Data int16_t grvecZ; // Gravity Vector Z Data int16_t res; // reserved uint32_t CRC; // CRC-32 } IMU_DATA; | Applicable for - D – Send Inertial Measurement Data |
| --- | --- | --- |

|  | typedef struct _UCDATA_SETINT3D { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (12) int16_t X; // X Data int16_t Y; // Y Data int16_t Z; // Z Data int16_t res; // reserved uint32_t CRC; // CRC-32 } UCDATA _SETINT3D; | Applicable for - OA – Send Accelerometer Offsets - OG – Send Gyroscope Offsets |
| --- | --- | --- |
|  | typedef struct _UCDATA_SETUINT3D { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (12) uint16_t X; // X Data uint16_t Y; // Y Data uint16_t Z; // Z Data int16_t res; // reserved uint32_t CRC; // CRC-32 } UCDATA _SETUINT3D; | Applicable for - GA – Send Accelerometer Gain Correction - GG – Send Gyroscope Gain Correction |
|  | typedef struct _ UCDATA _SETUINT { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (8) uint16_t Data; // Data int16_t res; // reserved uint32_t CRC; // CRC-32 } UCDATA _SETUINT; | Applicable for - RA – Send Acc. Offset Corr. On/Off - RG – Send Gyro Offset Corr. On/Off - QA – Send Acc. Gain Correction On/Off - QG – Send Gyro Gain Correction On/Off - P – Send Measurement Period |
|  | typedef struct _ UCDATA _REQPARS { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (4) uint32_t CRC; // CRC-32 } UCDATA _REQREQPARS; | Applicable for - FC – Write to Data Flash Completed - FE – Write to Data Flash Error |
|  | typedef struct _ UCDATA _IMUCONF { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (8) uint8_t B0; // Byte 0 uint8_t B1; // Byte 1 uint8_t B2; // Byte 2 uint8_t B3; // Byte 3 uint32_t CRC; // CRC-32 } UCDATA _IMUCONF; | Applicable for - C – Set IMU Configuration |
|  | typedef struct _ UCDATA _ERRMSG { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (8) uint16_t source; // Error Source Code uint16_t info; // Error Information Code uint32_t CRC; // CRC-32 } UCDATA _ ERRMSG; | Applicable for - E – Error Reporting |
|  | typedef struct _ UCDATA _ERRMSG { uint8_t cmd[2]; // Command Code (2 character) uint16_t length; // Payload Length (128) char text[124}; // Message Text uint32_t CRC; // CRC-32 } UCDATA _ ERRMSG; | Applicable for - M – Message |