MecanumMC Configuration

Hardware:

- NUCLEO-F401RE Microcontroller Panel

- 2 pcs two-channel DC Motor Controller: driving DC motors belonging to Mecanum wheels

  - two-wire PWM control on every motor

- 4 pcs AMS Omron AS5047U magnetic On-Axis Rotary Position Sensor

  - RPM measurement on the DC motor rear shafts: using quadrature AB signals

  - SPI control for parameter settings

- WizFi360 WiFi communication module connected to an UART line

  - for wireless remote control of the Mecanum wheel car

- Thermally compensated IMU module based on ST ASM330LHH sensor for inertial measurements

  - SPI control with interrupt

  - two-wire PWM control for thermal compensation

USART / UART Communication

UART2 (USART2)

Application:	Virtual COM Port (VCP) by using USB port of Nucleo STLINK

Mode:	Asynchronous

| Mode |  | Asynchronous |
| --- | --- | --- |
| Baud Rate | Bits/s | 460800 |
| Word Length |  | 8 |
| Parity |  | None |
| Stop Bits |  | 1 |
| Data Direction |  | Receive and Transmit |
| Over Sampling |  | 16 Samples |
| Interrupt |  | 7 |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| VPC_RX | PA2 | USART2_RX | n. a. | Very High | no |
| VPC_TX | PA3 | USART2_TX | n. a. | Very High | no |

UART6 (USART6)

Application:	RF communication by using WizFi360 WiFi module

Mode:	Asynchronous

| Mode |  | Asynchronous |
| --- | --- | --- |
| Baud Rate | Bits/s | 115200 |
| Word Length |  | 8 |
| Parity |  | None |
| Stop Bits |  | 1 |
| Data Direction |  | Receive and Transmit |
| Over Sampling |  | 16 Samples |
| Interrupt |  | 6 |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| RFCOM_RX | PA12 | USART6_RX | n. a. | Very High | no |
| RFCOM_TX | PC6 | USART6_TX | n. a. | Very High | no |

Timers

Timer10 (TIM10) 16-bit

Application: Base Loop Timer, Triggering AD conversion

| Clock Source | Activated | yes |
| --- | --- | --- |
| One Pulse Mode |  | off |
| Prescaler (PSC) | APB2 clock 84MHz | 84-1 |
| Counter Mode |  | Up |
| Counter Period (AutoReload Register) |  | 1000-1 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Freq. [kHz] | 1 |
| Global Interrupt |  | 3 |

Timer2 (TIM2) 32-bit

Application: RPS Measurement based on Quadrature A signal

| Clock Source |  | Internal Clock |
| --- | --- | --- |
| Channel 1 | PA15 | Input Capture direct mode |
| Channel 2 | PA10 | Input Capture direct mode |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84MHz | 0 |
| Counter Mode |  | Up |
| Counter Period (AutoReload Register) |  | 4294967295 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Frequency [MHz] | 84 |
| Global Interrupt |  | 2 |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| Input Capture Channel 1 |  |  |
| Polarity Selection |  | Rising Edge |
| IC Selection |  | Direct |
| Prescaler Division Ratio |  | No division |
| Input Filter (4 bits value) |  | 4 |
| Input Capture Channel 2 |  |  |
| Polarity Selection |  | Rising Edge |
| IC Selection |  | Direct |
| Prescaler Division Ratio |  | No division |
| Input Filter (4 bits value) |  | 4 |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| MWC_RPSA_RL | PA15 | TIM2_CH1 | n. a. | Very High | no |
| MWC_RPSA_RR | PB3 | TIM2_CH2 | n. a. | Very High | no |
| MWC_RPSB_RL | PC1 | GPIO_Input | n. a. | n. a. | no |
| MWC_RPSB_RR | PC0 | GPIO_Input | n. a. | n. a. | no |

Timer5 (TIM5) 32-bit

Application: RPS Measurement based on Quadrature A signal

| Clock Source |  | Internal Clock |
| --- | --- | --- |
| Channel 1 | PA0 | Input Capture direct mode |
| Channel 2 | PA1 | Input Capture direct mode |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84MHz | 0 |
| Counter Mode |  | Up |
| Counter Period (AutoReload Register) |  | 4294967295 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Frequency [MHz] | 84 |
| Global Interrupt |  | 2 |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| Input Capture Channel 1 |  |  |
| Polarity Selection |  | Rising Edge |
| IC Selection |  | Direct |
| Prescaler Division Ratio |  | No division |
| Input Filter (4 bits value) |  | 4 |
| Input Capture Channel 2 |  |  |
| Polarity Selection |  | Rising Edge |
| IC Selection |  | Direct |
| Prescaler Division Ratio |  | No division |
| Input Filter (4 bits value) |  | 4 |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| MWC_RPSA_FL | PA0 | TIM5_CH1 | n. a. | Very High | no |
| MWC_RPSA_FR | PA1 | TIM5_CH2 | n. a. | Very High | no |
| MWC_RPSB_FL | PA4 | GPIO_Input | n. a. | n. a. | no |
| MWC_RPSB_FR | PB0 | GPIO_Input | n. a. | n. a. | no |

Timer1 (TIM1) 16-bit

Application: DC Motor PWM Control

| Slave Mode |  | Disable |
| --- | --- | --- |
| Trigger Source |  | Disable |
| Clock Source |  | Internal Clock |
| Channel 1 | PA8 | PWM Generation CH1 |
| Channel 2 | PA9 | PWM Generation CH2 |
| Channel 3 | PA10 | PWM Generation CH3 |
| Channel 4 | PA11 | PWM Generation CH4 |
| Activate-Break-Input |  | Off |
| Use ETR as Clearing Source |  | Off |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84 MHz | 0 |
| Counter Mode |  | Center Aligned mode 1 |
| Counter Period (AutoReload Register) |  | 2048-1 |
| Internal Clock Division (CKD) |  | No Division |
| Repetition Counter |  | 0 |
| Auto-reload preload |  | Disable |
|  | Counting Freq. [kHz] | 41 015.625 |
|  | PWM frequency [kHz] | 20 507.8125 |
| Global Interrupt |  | - |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| Break And DeadTime management |  | n. a. |
| PWM Generation Channel 1 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| CH Idle State |  | Reset |
| PWM Generation Channel 2 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| CH Idle State |  | Reset |
| PWM Generation Channel 3 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| CH Idle State |  | Reset |
| PWM Generation Channel 4 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| CH Idle State |  | Reset |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| MWC_IN1_FL | PA8 | TIM1_CH1 | n. a. | High | no |
| MWC_IN2_FL | PA9 | TIM1_CH2 | n. a. | High | no |
| MWC_IN1_FR | PA10 | TIM1_CH3 | n. a. | High | no |
| MWC_IN2_FR | PA11 | TIM1_CH4 | n. a. | High | no |
| MWC_EN_FL | PB4 | GPIO_Output | Low | High | no |
| MWC_EN_FR | PB5 | GPIO_Output | Low | High | no |

Timer4 (TIM4) 16-bit

Application: DC Motor PWM Control

| Slave Mode |  | Disable |
| --- | --- | --- |
| Trigger Source |  | Disable |
| Internal Clock |  | On |
| Channel 1 | PB6 | PWM Generation CH1 |
| Channel 2 | PB7 | PWM Generation CH2 |
| Channel 3 | PB8 | PWM Generation CH3 |
| Channel 4 | PB9 | PWM Generation CH4 |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84 MHz | 0 |
| Counter Mode |  | Center Aligned mode 1 |
| Counter Period (AutoReload Register) |  | 2048-1 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
| Repetition Counter |  | 0 |
|  | Counting Freq. [kHz] | 41 015.625 |
|  | PWM frequency [kHz] | 20 507.8125 |
| Global Interrupt |  | - |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| PWM Generation Channel 1 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| PWM Generation Channel 2 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| PWM Generation Channel 3 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| PWM Generation Channel 4 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |

Signals associated with DC Motor Control

| Signal | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| MWC_IN1_RL | PB6 | TIM4_CH1 | n. a. | High | no |
| MWC_IN2_RL | PB7 | TIM4_CH2 | n. a. | High | no |
| MWC_IN1_RR | PB8 | TIM4_CH3 | n. a. | High | no |
| MWC_IN2_RR | PB9 | TIM4_CH4 | n. a. | High | no |
| MWC_EN_RL | PC9 | GPIO_Output | Low | High | no |
| MWC_EN_RR | PC8 | GPIO_Output | Low | High | no |

Timer3 (TIM3) 16-bit

Application: IMU Thermal Compensation PWM Control

| Slave Mode |  | Disable |
| --- | --- | --- |
| Trigger Source |  | Disable |
| Clock Source |  | Internal Clock |
| Channel 1 | PA6 | PWM Generation CH1 |
| Channel 2 | PA7 | PWM Generation CH2 |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84 MHz | 0 |
| Counter Mode |  | Center Aligned mode 1 |
| Counter Period (AutoReload Register) |  | 2048-1 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Freq. [kHz] | 41 015.625 |
|  | PWM frequency [kHz] | 20 507.8125 |
| Global Interrupt |  | - |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| PWM Generation Channel 1 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| PWM Generation Channel 2 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| IMU_TC1 | PA6 | TIM3_CH1 | n. a. | High | no |
| IMU_TC2 | PC7 | TIM3_CH2 | n. a. | High | no |

SPI Communication

SPI2

Application:	Handling AS5047U (AMS) 14-Bit On-Axis Magnetic Rotary Position Sensor

| Op. Mode |  | Full Duplex Master |
| --- | --- | --- |
| Hardware NSS Signal |  | Disable |
| Frame Format |  | Motorola |
| Data Size |  | 8 Bits |
| First Bit |  | MSB First |
| Prescaler (for Baud Rate) | APB1 clock 42 MHz | 8 |
|  | Baud Rate [MBits/s] | 5.250 |
| Clock Polarity (CPOL) |  | Low |
| Clock Phase (CPHA) |  | 2 Edge |
| CRC Calculation |  | Disabled |
| NSS Signal Type |  | Software |
| Global Interrupt |  | - |

| Signals | Portbit |  | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- | --- |
| RPSPI _SCK | PB10 |  | SPI2_SCK | n.a. | Very High | Ext. pulldown |
| RPSPI_MOSI | PC3 |  | SPI2_MOSI | n.a. | Very High | Ext. pulldown |
| RPSPI_MISO | PC2 |  | SPI2_MISO | n.a. | Very High | none |
| RPSPI_NSS_FL |  | PB1 | GPIO_Output | High | High | Ext. pullup |
| RPSPI_NSS_FR |  | PB13 | GPIO_Output | High | High | Ext. pullup |
| RPSPI_NSS_RL |  | PB14 | GPIO_Output | High | High | Ext. pullup |
| RPSPI_NSS_RR |  | PB15 | GPIO_Output | High | High | Ext. pullup |

SPI3

Application:	Handling ASM330LHH Inertial Measurement Unit (IMU)

| Op. Mode |  | Full Duplex Master |
| --- | --- | --- |
| Hardware NSS Signal |  | Disable |
| Frame Format |  | Motorola |
| Data Size |  | 8 Bits |
| First Bit |  | MSB First |
| Prescaler (for Baud Rate) | APB1 clock 42 MHz | 8 |
|  | Baud Rate [MBits/s] | 5.25 |
| Clock Polarity (CPOL) |  | High |
| Clock Phase (CPHA) |  | 2 Edge |
| CRC Calculation |  | Disabled |
| NSS Signal Type |  | Software |
| SPI3 Global Interrupt |  | - |
| DMA2 stream 0 global interrupt |  | 1 |
| DMA2 stream 5 global interrupt |  | 1 |

| DMA Settings |  |  |  |
| --- | --- | --- | --- |
| DMA Request | Stream | Direction | Priority |
| SPI3_RX | DMA1 Stream 0 | Peripheral To Memory | High |
| SPI3_TX | DMA1 Stream 5 | Memory To Peripheral | High |

| Signals | Portbit |  | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- | --- |
| IMU_SCK | PC10 |  | SPI3_SCK | n.a. | Very High | Ext. pullup |
| IMU _MOSI | PC12 |  | SPI3_MOSI | n.a. | Very High | no |
| IMU _MISO | PC11 |  | SPI3_MISO | n.a. | Very High | Ext. pullup |
| IMU _NSS |  | PD2 | GPIO_Output | High | High | Ext. pullup |
| IMU _INT |  | PC4 | GPIO_EXTI4 | n.a. | n.a. | no |
| IMU _RST |  | PC5 | GPIO_Output | High | Low | no |

Analog Peripherals

ADC1

Application:	Analog Voltage Measurement

| Channel | Function | Port I/O | Designation |
| --- | --- | --- | --- |
| IN7 | Motor Voltage | PA7 | PSENS_AIN |

| Clock Prescaler |  | PCLK2 divided by 4 |
| --- | --- | --- |
| Resolution |  | 12 bits (15 ADC Clock cycles) |
| Data Alignment |  | Right alignment |
| Scan Conversion Mode |  | Enabled |
| Continuous Conversion Mode |  | Disabled |
| Discontinuous Conversion Mode |  | Disabled |
| DMA Continuous Request |  | Disabled |
| End of Conversion Selection | EOC flag at the | end of all conversions |
| ADC Injected Conversion Mode |  |  |
| Number of Conversion |  | 1 |
| External Trigger Source | Injected Conversion | Timer 4 Trigger Out event |
| External Trigger Edge |  | Rising Edge |
| Injected Conversion Mode |  | None |

| Injected Rank | Channel | Sampling Time | Injected Offset |
| --- | --- | --- | --- |
| 1 | IN7 | 15 cycles | 0 |

| ADC1 global Interrupt |  | 5 |
| --- | --- | --- |

GPIO

GPIO_EXTI

| Port bit | Signal | EXTIn | INT | Mode | Function |
| --- | --- | --- | --- | --- | --- |
| PC4 | IMU_INT | 4 | 4 | Rising Edge | IMU Sensor Interrupt signal |
| PC13 | BLUE_SW | 13 | 8 | Rising/Falling Edge | Nucleo board Blue Switch |

GPIO_Input

| Port bit | Signal | Pull Up /Down | Function |
| --- | --- | --- | --- |
| PA4 | MWC_RPSB_FL | no | Quadrature B signal for RPS measurement – Front Left |
| PB0 | MWC_RPSB_FR | no | Quadrature B signal for RPS measurement – Front Right |
| PC1 | MWC_RPSB_RL | no | Quadrature B signal for RPS measurement – Rear Left |
| PC0 | MWC_RPSB_RR | no | Quadrature B signal for RPS measurement – Rear Right |

GPIO_Output

| Port bit | Signal | Pull Up /Down | Max. output speed | Level | Function |
| --- | --- | --- | --- | --- | --- |
| PD2 | SENS_NSS | ext up |  | Low | IMU Sensor SPI Select (negated) |
| PA5 | GREEN_LED | no | Low | Low | Nucleo board Green LED On/Off |
| PC5 | SENS_RST | no | Low | Low | IMU Sensor Reset signal |
| PB1 | RPSPI_NSS_FL | ext up | Low | Low | RPS Sensor SPI Select - Front Left |
| PB15 | RPSPI_NSS_RR | ext up | Low | Low | RPS Sensor SPI Select - Front Right |
| PB14 | RPSPI_NSS_RL | ext up | Low | Low | RPS Sensor SPI Select - Rear Left |
| PB13 | RPSPI_NSS_FR | ext up | Low | Low | RPS Sensor SPI Select - Rear Right |
| PB4 | MWC_EN_FL | no | Low | Low | DC Motor Controller Enable - FL |
| PB5 | MWC_EN_FR | no | Low | Low | DC Motor Controller Enable - FR |
| PC9 | MWC_EN_RL | no | Low | Low | DC Motor Controller Enable - RL |
| PC8 | MWC_EN_RR | no | Low | Low | DC Motor Controller Enable - RR |
| PB2 | USER_LED1 | no | Low | Low | User LED (YELLOW) |
| PB12 | USER_LED2 | no | Low | Low | User LED (RED) |

NVIC

| Interrupt | Usage | Priority |
| --- | --- | --- |
| DMA2 stream 0 global interrupt | IMU SPI DMA Interrupt | 1 |
| DMA2 stream 5 global interrupt | IMU SPI DMA Interrupt | 1 |
| TIM2 global interrupt | RPS Measurement | 2 |
| TIM5 global interrupt | RPS Measurement | 2 |
| TIM1 update and TIM10 global interrupt | Realizing basic program cycle period | 3 |
| EXTI line4 interrupt | IMU Interrupt | 4 |
| ADC1 global Interrupt | Analog Voltage Measurement | 5 |
| USART6 global interrupt | Wireless communication | 6 |
| USART2 global interrupt | Virtual COM Port | 7 |
| EXTI Line [15:10] interrupt | User switch | 8 |