SensActDCMctrl Configuration

Hardware:

- SensActNucleo64 Demonstration Panel

- NUCLEO-F401RE Microcontroller Panel

- X-NUCLEO-IHM04A1 DC Motor Controller (Arduino connector)

- Two AMS AS5047U magnetic On-Axis Rotary Position Sensor

  - RPM measurement on the rear shaft: Quadrature AB signals

  - Absolute Angle measurement on the front shaft: SPI

USART / UART Communication

UART2 (USART2)

Application:	Virtual COM Port (VCP) by using USB port of Nucleo STLINK

Mode:	Asynchronous

| Mode |  | Asynchronous |
| --- | --- | --- |
| Baud Rate | Bits/s | 460800 |
| Word Length |  | 8 |
| Parity |  | None |
| Stop Bits |  | 1 |
| Data Direction |  | Receive and Transmit |
| Over Sampling |  | 16 Samples |
| Interrupt |  | 7 |

Timers

Timer10 (TIM10) 16-bit

Application: Base Loop Timer

| Clock Source | Activated | yes |
| --- | --- | --- |
| One Pulse Mode |  | off |
| Prescaler (PSC) | APB2 clock 84MHz | 84-1 |
| Counter Mode |  | Up |
| Counter Period (AutoReload Register) |  | 1000-1 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Freq. [kHz] | 1 |
| Global Interrupt |  | 2 |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |

Timer2 (TIM2) 32-bit

Application: RPS Measurement based on Quadrature A signal

| Clock Source |  | Internal Clock |
| --- | --- | --- |
| Channel 1 |  | Input Capture direct mode |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84MHz | 0 |
| Counter Mode |  | Up |
| Counter Period (AutoReload Register) |  | 4294967295 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Frequency [MHz] | 84 |
| Global Interrupt |  | 1 |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| Input Capture Channel 1 |  |  |
| Polarity Selection |  | Rising Edge |
| IC Selection |  | Direct |
| Prescaler Division Ratio |  | No division |
| Input Filter (4 bits value) |  | 4 |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| RPSM_QA | PA15 | TIM2_CH1 | n. a. | Very High | no |

Timer3 (TIM3) 16-bit

Application: DC Motor PWM Control

| Clock Source |  | Internal Clock |
| --- | --- | --- |
| Channel 1 |  | PWM Generation CH1 |
| Channel 2 |  | PWM Generation CH2 |
| One Pulse Mode |  | Off |
| Prescaler (PSC) | APB1 clock 84 MHz | 0 |
| Counter Mode |  | Center Aligned mode 1 |
| Counter Period (AutoReload Register) |  | 2048-1 |
| Internal Clock Division (CKD) |  | No Division |
| Auto-reload preload |  | Disable |
|  | Counting Freq. [kHz] | 41 015.625 |
|  | PWM frequency [kHz] | 20 507.8125 |
| Global Interrupt |  | - |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Reset |
| PWM Generation Channel 1 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |
| PWM Generation Channel 2 |  |  |
| Mode |  | PWM mode 1 |
| Pulse (32 bits value) |  | 0 |
| Output Compare preload |  | Enable |
| Fast Mode |  | Disable |
| CH Polarity |  | High |

| Signals | Portbit | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- |
| DCMC_IN1_A | PB4 | TIM3_CH1 | n. a. | High | no |
| DSMC_IN2_A | PB5 | TIM3_CH2 | n. a. | High | no |

Timer4 (TIM4) 16-bit

Application: Triggering ADC Measurement

| Clock Source |  | Internal Clock |
| --- | --- | --- |
| Prescaler | APB1 clock 84 MHz | 84-1 |
| Counter Mode |  | Up |
| Counter Period (AutoReload Register) |  | 100-1 |
| Internal Clock Division (CKD) |  | No Division |
| auto-reload preload |  | Disable |
|  | Counting Frequency [kHz] | 10 |
| Trigger Output (TRGO) Parameters |  |  |
| Master/Slave Mode (MSM bit) |  | Disable |
| Trigger Event Selection |  | Update Event |
| Global Interrupt |  | - |

| Triggered Peripheral | ADC: start measurement | 10 kHz |
| --- | --- | --- |

SPI Communication

SPI2

Application:	Handling AS4047U (AMS) 14-Bit On-Axis Magnetic Rotary Position Sensor

| Op. Mode |  | Full Duplex Master |
| --- | --- | --- |
| Hardware NSS Signal |  | Disable |
| Frame Format |  | Motorola |
| Data Size |  | 8 Bits |
| First Bit |  | MSB First |
| Prescaler (for Baud Rate) | APB1 clock 42 MHz | 8 |
|  | Baud Rate [MBits/s] | 5.250 |
| Clock Polarity (CPOL) |  | Low |
| Clock Phase (CPHA) |  | 2 Edge |
| CRC Calculation |  | Disabled |
| NSS Signal Type |  | Software |
| Global Interrupt |  | - |

| Signals | Portbit |  | Per. function | Default | Max. output speed | Pullup/down |
| --- | --- | --- | --- | --- | --- | --- |
| ASENS _SCK | PB10 |  | SPI2_SCK | n.a. | Very High | Ext. pullup |
| ASENS _MOSI | PC3 |  | SPI2_MOSI | n.a. | Very High | no |
| ASENS_MISO | PC2 |  | SPI2_MISO | n.a. | Very High | no |
| ANGS _NSS |  | PA6 | GPIO_Output | High | High | no |
| RPMS _NSS |  | PC0 | GPIO_Output | High | High | no |

Analog Peripherals

ADC1

Application:	Analog Voltage Measurement

| Channel | Function | Port I/O | Designation |
| --- | --- | --- | --- |
| IN14 | Potentiometer 1 Voltage | PC4 | VPOT1_IN14 |
| IN15 | Potentiometer 1 Voltage | PC5 | VPOT2_IN14 |
| IN7 | Motor Voltage | PA7 | PSENSE_IN7 |

| Clock Prescaler |  | PCLK2 divided by 4 |
| --- | --- | --- |
| Resolution |  | 12 bits (15 ADC Clock cycles) |
| Data Alignment |  | Right alignment |
| Scan Conversion Mode |  | Enabled |
| Continuous Conversion Mode |  | Disabled |
| Discontinuous Conversion Mode |  | Disabled |
| DMA Continuous Request |  | Disabled |
| End of Conversion Selection | EOC flag at the | end of all conversions |
| ADC Injected Conversion Mode |  |  |
| Number of Conversion |  | 3 |
| External Trigger Source |  | Timer 4 Trigger Out event |
| External Trigger Edge |  | Rising Edge |
| Injected Conversion Mode |  | None |

| Injected Rank | Channel | Sampling Time | Injected Offset |
| --- | --- | --- | --- |
| 1 | IN14 | 28 cycles | 0 |
| 2 | IN15 | 28 cycles | 0 |
| 3 | IN7 | 28 cycles | 0 |

| ADC1 global Interrupt |  | 7 |
| --- | --- | --- |

GPIO

GPIO_EXTI

| Port bit | Function | Signal | INT | Mode |
| --- | --- | --- | --- | --- |
| PC9 | GPIO_EXTI9 | USR_SW1 | 8 | Rising/Falling Edge |
| PA11 | GPIO_EXTI11 | USR_SW2 | 8 | Rising/Falling Edge |
| PB12 | GPIO_EXTI12 | USR_SW3 | 8 | Rising/Falling Edge |
| PB15 | GPIO_EXTI15 | USR_SW4 | 8 | Rising/Falling Edge |
| PB14 | GPIO_EXTI14 | USR_SW5 | 8 | Rising/Falling Edge |
| PB13 | GPIO_EXTI13 | USR_SW6 | 8 | Rising/Falling Edge |

GPIO

| Port bit | Function | Signal | Mode | Max. output speed | Level |
| --- | --- | --- | --- | --- | --- |
| PA0 | Input | DCMC_IN1_B | No PullUp/Down | n.a. | - |
| PA1 | Input | DCMC_IN2_B | No PullUp/Down | n.a. | - |
| PA4 | Output Pushpull | RPSM_QB | No PullUp/Down | High | Low |
| PA6 | Output Pushpull | ANGS_NSS | No PullUp/Down | High | High |
| PA8 | Output Pushpull | YEL5_LED | No PullUp/Down | Low | Low |
| PA9 | Output Pushpull | YEL6_LED | No PullUp/Down | Low | Low |
| PA10 | Output Pushpull | DCMC_EN_A | No PullUp/Down | Low | Low |
| PA12 | Output Pushpull | YEL3_LED | No PullUp/Down | Low | Low |
| PB0 | Output Pushpull | PS_LED_R | No PullUp/Down | Low | Low |
| PB1 | Output Pushpull | YEL1_LED | No PullUp/Down | Low | Low |
| PB2 | Output Pushpull | YEL2_LED | No PullUp/Down | Low | Low |
| PB3 | Output Pushpull | PS_LED_G | No PullUp/Down | Low | Low |
| PC0 | Output Pushpull | RPMS_NSS | No PullUp/Down | High | High |
| PC1 | Output Pushpull | DCMC_EN_B | No PullUp/Down | Low | Low |
| PC6 | Output Pushpull | RED_LED | No PullUp/Down | Low | Low |
| PC7 | Output Pushpull | GREEN_LED | No PullUp/Down | Low | Low |
| PC8 | Output Pushpull | YEL7_LED | No PullUp/Down | Low | Low |
| PC13 | Output Pushpull | YEL4_LED | No PullUp/Down | Low | Low |

NVIC

| Interrupt | Usage | Priority |
| --- | --- | --- |
| TIM2 global interrupt | RPS Measurement | 1 |
| TIM1 update and TIM10 global interrupt | Realizing basic program cycle period | 2 |
| ADC1 global interrupts | ADC Measurement | 4 |
| USART2 global interrupt | Virtual COM Port | 5 |
| EXTI Line [9:5] interrupt | User switches | 6 |
| EXTI Line [15:10] interrupt | User switches | 6 |